## Digital Electronics and Logic Design

Introduction

Digital electronics is a critical area of electrical and electronics engineering, forming the core of most modern electronic devices and systems. It encompasses the study and application of digital circuits, which utilize discrete signal levels to represent information. Logic design is a key aspect of this field, focusing on the creation and optimization of digital circuits for various practical applications. Mastering these concepts is essential for careers in computing, telecommunications, automation, and consumer electronics, where digital electronics and logic design play a pivotal role in the development of innovative technologies and solutions.

### What is logic design in digital electronics?

Logic design is the process of creating circuits that perform specific functions based on binary logic. It involves using logic gates and combinational and sequential circuits to create complex digital systems that can perform a variety of tasks.

### Why is digital electronics important?

Digital electronics is crucial because it forms the foundation of modern computing and communication systems. It enables the development of complex electronic devices and systems that are faster, more efficient, and more reliable than their analog counterparts.

### Number System and Representation

# Binary Representations in Digital Logic

In the digital system of logic, binary forms have core functions with respect to data processing. Binary numbers systems in which only two states are possible-0 and 1 are used in digital circuits for mathematical computations for storing information as well as information transmission. Such techniques include sign and magnitude, one’s complement and two’s complement and etc which are used to represent both positive and negative numbers in digital systems to enable arithmetic and logic operations to work full well.

## What is Binary Number System?

Binary is a base-2 number system that uses two states 0 and 1 to represent a number. We can also call it to be a true state and a false state. A binary number is built the same way as we build the normal decimal number.

For example, a decimal number 45 can be represented as 4*10^1+5*10^0 = 40+5

Now in binary 45 is represented as 101101. As we have powers of 10 in decimal number similarly there are powers of 2 in binary numbers. Hence 45 which is 101101 in binary can be represented as:

2^0*1+2^1*0+2^2*1+2^3*1+2^4*0+2^5*1 = 45

The binary number is traversed from left to right.

## Types of Binary Representations in Digital Electronics

### Sign and Magnitude Representation

There are many ways for representing negative integers. One of the way is sign-magnitude. This system uses one bit to indicate the sign. Mathematical numbers are generally made up of a sign and a value. The sign indicates whether the number is positive, (+) or negative, (–) while the value indicates the size of the number.

For example 13, +256 or -574. Presenting numbers in this way is called sign-magnitude representation since the left most digit can be used to indicate the sign and the remaining digits the magnitude or value of the number.

For example 0101101 represents +45 and 1101101 represents -45 if 6 digits of a binary number are considered and the leftmost digit represents the sign.

But a problem with the sign-magnitude method is that it can result in the possibility of two different bit patterns having the same binary value. For example, +0 and -0 would be 0000 and 1000 respectively as a signed 4-bit binary number. So using this method there can be two representations for zero, a positive zero 0000 and also a negative zero 1000 which can cause big complications for computers and digital systems.

The 2 complement notations used to represent signed magnitude numbers are:

### One’s complement

One’s Complement is a method which can be used to represent negative binary numbers in a signed binary number system. In one’s complement, positive numbers remain unchanged as before.

Negative numbers however, are represented by taking the one’s complement of the unsigned positive number. Since positive numbers always start with a 0, the complement will always start with a 1 to indicate a negative number.

The one’s complement of a negative binary number is the complement of its positive, so to take the one’s complement of a binary number, all we need to do is subtract 1’s equal to the number of digits present in the number from that number. This can also be achieved by just interchanging the digits of the number. Thus the one’s complement of 1 is 0 and vice versa.

For example One’s Complement of 1010100:

1111111 -1010100 0101011

The one’s complement of number can also be obtained by just interchanging the digits of the binary number.

### Two’s complement

Two’s Complement is another method like one’s complement form, which we can use to represent negative binary numbers in a signed binary number system. In two’s complement, the positive numbers are exactly the same as before for unsigned binary numbers. A negative number, however, is represented by a binary number, which when added to its corresponding positive equivalent results in zero.

In two’s complement representation, a negative number is the 2’s complement of its positive number. If the subtraction of two numbers is X – Y then it can be represented as X + (2’s complement of Y).

The two’s complement is one’s complement + 1 of a number in binary.

The main advantage of two’s complement over the previous one’s complement is that there is no double-zero problem, and it is a lot easier to generate the two’s complement of a signed binary number. In two’s complement arithmetic operations are relatively easier to perform when the numbers are represented in the two’s complement format.

For example to represent -27

27 in binary is: 00011011

11111111 -00011011 11100100 <– 1’s Complement +1 11100101 <– The 2’s Complement

The above 2 are the formats that can be too long practically. So real number representations are used.

### Real Number Representations

The goal is to represent a number with a decimal point in binary using the form. IEEE 754 standard defines how to encode a real number. This standard offers a way to code a number using 32 bits (as well as 64 bits), and defines three components:

- The plus/minus sign is represented by one bit, the highest-weighted bit (furthest to the left).

- The exponent is encoded using 8 bits (11 bits in 64 bit representation) immediately after the sign.

- The mantissa (the bits after the decimal point) with the remaining 23 bits(52 bits in 64 bit representation).

## Analog and Digital Signals

- Analog Signals: Analog signals are of analogue type and they are characterized by being continuous in form and fluctuate smoothly. They can have a large range of applications and are used in simple radio, audio and video systems.

- Digital Signals: Digital signals are actually analogue and are spaced a good distance apart from each other and are basically any kind of data in the form of 0’s and 1’s. They are used in microprocessors, computers, telecommunication and in other digital products and devices.

## Advantages

### Binary System

- Simplicity in hardware design.

- Error detection and correction are not as complex as the other error control techniques.

- Effective for computer based calculations as well as computerized data archiving.

Sign and Magnitude

- They are easily understandable as well as easy to be implemented.

### One’s Complement and Two’s Complement

- This in turn helps to perform arithmetic operations easily and does not result to what we call dual zeroes.

## Disadvantages

### Sign and Magnitude

- Zero creates problem to various operations because it has two symbols.

### One’s Complement

- Dual zeroes issue.

- More complex arithmetic operations.

### Two’s Complement

- At least apparently more complex to comprehend at first, but in most instances, more effective overall.

## Applications

- Binary Numbers: Employed in all up-to-date computer-based processing and transmission methods.

- Sign and Magnitude: Not used actively in practice for couple of decades but still can be useful in simple educational settings.

- One’s Complement: Used in some of the older systems and networking protocols.

- Two’s Complement: Common in today’s computers performing calculations because of its effectiveness and ease in computation.

## Conclusion

Binary representation is one of the core aspects of digital systems right from computing. Of the three representation methods -sign and magnitude, one’s complement, and two’s complement, each has its appropriateness concerning the complexity of summing operations. Two’s complement is preferred in most of the modern systems because it has some benefits over the other methods; arithmetic operations becomes much easier with it and there are no dual zeroes.

# Number System and Base Conversions

Electronic and Digital systems may use a variety of different number systems, (e.g. Decimal, Hexadecimal, Octal, Binary), or even Duodecimal or less well known but better named Uncial. All the other bases other than Decimal result from computer usage. Uncial (named from Latin for 1/12 “uncia” the base twelve analogue of Decimal from the Latin word for 1/10 “decima”).

## What is Number System ?

A number system is a method to represent numbers mathematically. It can use arithmetic operations to represent every number uniquely. To represent a number, it requires a base or radix.

## Types of Number System

There are four common types of number systems based on the radix or base of the number :

- Decimal number system or Base 10 number system

- Binary number system or Base 2 number system

- Octal number system or Base 8 number system

- Hexadecimal number system or Base 16 number system

## Number System Conversion Methods

A number N in base or radix b can be written as:

(N)b = dn-1 dn-2 -- -- -- -- d1 d0 . d-1 d-2 -- -- -- -- d-m

In the above, dn-1 to d0 is the integer part, then follows a radix point, and then d-1 to d-m is the fractional part.

dn-1 = Most significant bit (MSB) 
d-m = Least significant bit (LSB)

## 1. Decimal to Binary Number System

To convert from decimal to binary, start dividing decimal number by 2, and whatever the reminder getting, writing down from bottom to top, and that will be the binary number representation of the decimal number. And the number contains fractional part, then multiply 2 in the fractional part.

### Example

(10.25)10

Note: Keep multiplying the fractional part with 2 until decimal part 0.00 is obtained. 
(0.25)10 = (0.01)2

Answer: (10.25)10 = (1010.01)2

## 2. Binary to Decimal Number System

To convert from binary to decimal, start multiplying the exponent of 2 with each digit of the number in decreasing order. If the number contains fractional part then will divide it by the exponent of 2.

### Example

(1010.01)2 
1x23 + 0x22 + 1x21+ 0x20 + 0x2 -1 + 1x2 -2 = 8+0+2+0+0+0.25 = 10.25 
(1010.01)2 = (10.25)10

## 3. Decimal to Octal Number System

To convert from decimal to octal, start dividing decimal number by 8, and whatever the reminder getting, writing down from bottom to top, and that will be the octal number representation of the decimal number. And the number contains fractional part, then multiply 8 in the fractional part.

### Example

(10.25)10 
(10)10 = (12)8 
Fractional part: 
0.25 x 8 = 2.00

Note: Keep multiplying the fractional part with 8 until decimal part .00 is obtained. 
(.25)10 = (.2)8

Answer: (10.25)10 = (12.2)8

## 4. Octal to Decimal Number System

To convert from octal to decimal, start multiplying the exponent of 8 with each digit of the number in decreasing order. If the number contains fractional part, then will divide it by the exponent of 8.

### Example

(12.2)8
1 x 81 + 2 x 80 +2 x 8-1 = 8+2+0.25 = 10.25 
(12.2)8 = (10.25)10

## 5. Hexadecimal to Binary Number System

To convert from Hexadecimal to Binary, write the 4-bit binary equivalent of hexadecimal.

### Example

(3A)16 = (00111010)2

## 6. Binary to Hexadecimal Number System

To convert from Binary to Hexadecimal, start grouping the bits in groups of 4 from the right-end and write the equivalent hexadecimal for the 4-bit binary. Add extra 0’s on the left to adjust the groups.

### Example

1111011011
0011 1101 1011
(001111011011 )2 = (3DB)16

## 7. Binary to Octal Number System

To convert from binary to octal, start grouping the bits in groups of 3 from the right end and write the equivalent octal for the 3-bit binary. Add 0’s on the left to adjust the groups.

### Example

111101101

111 101 101

(111101101)2 = (755)8

## Conclusion

In the conclusion, number system provides a representation of a number with respect to the given base or radix. Basically, we have common types of number system named as binary number system, decimal number system, octal number system and hexadecimal number system.

# Introduction of Floating Point Representation

Last Updated : 17 May, 2023

1. To convert the floating point into decimal, we have 3 elements in a 32-bit floating point representation: 
    i) Sign 
    ii) Exponent 
    iii) Mantissa

- Sign bit is the first bit of the binary representation. ‘1’ implies negative number and ‘0’ implies positive number. 
Example: 11000001110100000000000000000000 This is negative number.

- Exponent is decided by the next 8 bits of binary representation. 127 is the unique number for 32 bit floating point representation. It is known as bias. It is determined by 2k-1 -1 where ‘k’ is the number of bits in exponent field.

There are 3 exponent bits in 8-bit representation and 8 exponent bits in 32-bit representation.

Thus

bias = 3 for 8 bit conversion (23-1 -1 = 4-1 = 3) 
bias = 127 for 32 bit conversion. (28-1 -1 = 128-1 = 127)

Example: 01000001110100000000000000000000 
10000011 = (131)10 
131-127 = 4

Hence the exponent of 2 will be 4 i.e. 24 = 16.

- Mantissa is calculated from the remaining 23 bits of the binary representation. It consists of ‘1’ and a fractional part which is determined by:

Example:

01000001110100000000000000000000

The fractional part of mantissa is given by:

1*(1/2) + 0*(1/4) + 1*(1/8) + 0*(1/16) +……… = 0.625

Thus the mantissa will be 1 + 0.625 = 1.625

The decimal number hence given as: Sign*Exponent*Mantissa = (-1)0*(16)*(1.625) = 26

2. To convert the decimal into floating point, we have 3 elements in a 32-bit floating point representation: 
    i) Sign (MSB) 
    ii) Exponent (8 bits after MSB) 
    iii) Mantissa (Remaining 23 bits)

- Sign bit is the first bit of the binary representation. ‘1’ implies negative number and ‘0’ implies positive number. 
Example: To convert -17 into 32-bit floating point representation Sign bit = 1

- Exponent is decided by the nearest smaller or equal to 2n number. For 17, 16 is the nearest 2n. Hence the exponent of 2 will be 4 since 24 = 16. 127 is the unique number for 32 bit floating point representation. It is known as bias. It is determined by 2k-1 -1 where ‘k’ is the number of bits in exponent field.

Thus bias = 127 for 32 bit. (28-1 -1 = 128-1 = 127)

Now, 127 + 4 = 131 i.e. 10000011 in binary representation.

- Mantissa: 17 in binary = 10001.

Move the binary point so that there is only one bit from the left. Adjust the exponent of 2 so that the value does not change. This is normalizing the number. 1.0001 x 24. Now, consider the fractional part and represented as 23 bits by adding zeros.

00010000000000000000000

### Advantages:

Wide range of values: Floating factor illustration lets in for a extensive variety of values to be represented, along with very massive and really small numbers.

Precision: Floating factor illustration offers excessive precision, that is important for medical and engineering calculations.

Compatibility: Floating point illustration is extensively used in computer structures, making it well matched with a extensive variety of software and hardware.

Easy to use: Most programming languages offer integrated guide for floating factor illustration, making it smooth to use and control in laptop programs.

### Disadvantages:

Complexity: Floating factor illustration is complex and can be tough to understand, mainly for folks that aren’t acquainted with the underlying mathematics.

Rounding errors: Floating factor illustration can result in rounding mistakes, where the real price of a number of is barely extraordinary from its illustration inside the computer.

Speed: Floating factor operations can be slower than integer operations, particularly on older or much less powerful hardware.

Limited precision: Despite its excessive precision, floating factor representation has a restrained number of sizeable digits, which could restrict its usefulness in some programs.